USA, GPS MODULE (OEM version)

ISHTRONICS, Inc.

3620 Kennesaw, N. Ind. Pkwy

Kennesaw, GA 30144

Tel: (770) 590-1822  Fax: (770) 770-422-8408 e-mail: info@ishtronics.com

| TITLE: CERTIFICATION |  | SPECIFICATION |  |  | PART NO: 1100-1000 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | DOCUMENT NO: TFA0001 |
|  |  |  |  |  | PAGE: 1 of 4 |
|  | 1. Electrical Characteristics |  |  |  |  |
|  | 1.1 General |  | Frequency | L1,1575.42MHz |  |
|  |  |  | C/A code | 1.023 MHz chip rate |  |
|  |  |  | Channels | 12 |  |
|  | 1.2 Accuracy |  | Position | 25 meters CEP without SA |  |
|  |  |  | Velocity | 0.1 meters/second, without SA |  |
|  |  |  | Time | 1 microsecond synchronized to GPS time |  |
|  | 1.3 DGPS Accuracy |  | Position | 1 to 5 meters, typical |  |
|  |  |  | Velocity | 0.05 meters/second, typical |  |
|  | 1.4 Datum |  | WGS-84 |  |  |
|  | 1.5 Acquisition Rate |  | Reacquisition | 0.1 sec., average |  |
|  |  |  | Cold start | 48 sec., average |  |
|  |  |  | Warm start | 38 sec., average |  |
|  |  |  | Hot start | 8 sec., average |  |
|  | 1.6 Dynamic Condition |  | Altitude | 18,000 meters (60,000 Feet) max. |  |
|  |  |  | Velocity | 515 meters/sec.(1000 Knots) max. |  |
|  |  |  | Acceleration | 4 g., max. |  |
|  |  |  | Jerk | 20 meters/sec.3 max. |  |
|  | 1.7 Power |  | Main Power | +5Vdc±5% |  |
|  |  |  | Supply Current | 180 mA typical |  |
|  |  |  | Backup Power | +2.5V to 4.5V |  |
|  |  |  | Backup Current | 10µA typical |  |
|  | 1.8 External Reset |  | Active low input |  |  |
|  | 1.9 Serial Port |  | Electrical interface | Two full duplex serial communication |  |
|  |  |  | Protocol | Design-in binary and NMEA-0183, Version 2.00 with a baud rate selection |  |
|  |  |  | NMEA output | GGA,GLL,GSA,GSV,RMC and VTG(on customer request)Default all 6 NMEA |  |
|  |  |  |  | (Baud Rate :4800) |  |
|  |  |  | DGPS protocol | RTCM SC-104, version 2.00, type1,2 and 9 |  |
|  | 1.10 Time-1PPS Pulse |  | Level | TTL |  |
|  |  |  | Pulse duration | 100 ms |  |
|  |  |  | Time reference | At the pulse negative edge |  |
|  |  |  | Measurements | Aligned to GPS second, ± 1µ sec. |  |
|  | 2. Environmental Characteristics |  |  |  |  |
|  | 2.1Temperature |  | Operating range | - 40 C to + 85 C |  |
|  |  |  | Storage range | - 55 C to +100 C |  |

USA, Canada and South America

ISHTRONICS, Inc.

3620 Kennesaw, N. Ind. Pkwy

Kennesaw, GA 30144

Tel: (770) 590-1822  Fax: (770) 422-8408 e-mail: : info@ishtronics.com

| TITLE: CERTIFICATION |  | SPECIFICATION |  |  |  | PART NO: 1100-1000 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DOCUMENT NO: TFA0001 |
|  |  |  |  |  |  | PAGE: 2 of 4 |
|  | 2.2 Physical characteristics |  | Length |  | 72 mm |  |
|  |  |  | Width |  | 42 mm |  |
|  |  |  | Height |  | 12 mm |  |
|  |  |  | Weight |  | 23 gm |  |
|  |  |  | Antenna connector |  | MCX type |  |
|  |  |  | Interface connector |  | 20-pin straight header, 2 mm |  |
|  | 3. Antenna |  | On board LNA design, either Passive or Active Antenna |  |  |  |
|  | 4. Physical Dimensions |  |  |  |  |  |

USA, Canada and South America

ISHTRONICS, Inc.

3620 Kennesaw, N. Ind. Pkwy

Kennesaw, GA 30144

Tel: (770) 590-1822  Fax: (770) 422-8408 e-mail: info@ishtronics.com

| TITLE: CERTIFICATION |  |  |  |  |  |  |  | SPECIFICATION |  | PART NO: 1100-1000 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | DOCUMENT NO: TFA0001 |  |  |
|  |  |  |  |  |  |  |  |  |  | PAGE: 3 of 4 |  |  |
| 5. Connector Layout Top View |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 |  |  |  |  | #2 |  |  |  |  |  |
|  |  | #3 |  |  |  |  | #4 |  |  |  |  |  |
|  |  | #5 |  |  |  |  | #6 |  |  |  |  |  |
|  |  | #7 |  |  |  |  | #8 |  |  |  |  |  |
|  |  | #9 |  |  |  |  | #10 |  |  |  |  |  |
|  |  | #11 |  |  |  |  | #12 |  |  |  |  |  |
|  |  | #13 |  |  |  |  | #14 |  |  |  |  |  |
|  |  | #15 |  |  |  |  | #16 |  |  |  |  |  |
|  |  | #17 |  |  |  |  | #18 |  |  |  |  |  |
|  |  | #19 |  |  |  |  | #20 |  |  |  |  |  |
| Pin-out of the 20-pin digital interface connector |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pin # |  | Name |  |  |  |  |  | Description |  |  |  |
|  | 1 |  | VANT |  |  |  |  |  | +5V Antenna DC Voltage Input |  |  |  |
|  | 2 |  | VCC |  |  |  |  |  | +5V DC Power Input |  |  |  |
|  | 3 |  | BAT |  |  |  |  |  | Battery Backup Power Input |  |  |  |
|  | 4 |  | VDD |  |  |  |  |  | +3.3V DC Power Input |  |  |  |
|  | 5 |  | PBRES |  |  |  |  |  | Push Button Reset Input, Active Low |  |  |  |
|  | 6 |  | RESERVED |  |  |  |  |  | Reserved |  |  |  |
|  | 7 |  | RESERVED |  |  |  |  |  | Reserved |  |  |  |
|  | 8 |  | RESERVED |  |  |  |  |  | Reserved |  |  |  |
|  | 9 |  | RESERVED |  |  |  |  |  | Reserved |  |  |  |
|  | 10 |  | GND |  |  |  |  |  | Ground |  |  |  |
|  | 11 |  | TXA |  |  |  |  |  | Host Serial Data Output A |  |  |  |
|  | 12 |  | RXA |  |  |  |  |  | Host Serial Data Input A |  |  |  |
|  | 13 |  | GND |  |  |  |  |  | Ground |  |  |  |
|  | 14 |  | TXB |  |  |  |  |  | Aux. Serial Data Output B |  |  |  |
|  | 15 |  | RXB |  |  |  |  |  | Aux. Serial Data Input B (DGPS) |  |  |  |
|  | 16 |  | GND |  |  |  |  |  | Ground |  |  |  |
|  | 17 |  | RESERVED |  |  |  |  |  | Reserved |  |  |  |
|  | 18 |  | GND |  |  |  |  |  | Ground |  |  |  |
|  | 19 |  | TIMEMARK |  |  |  |  |  | 1PPS Time Mark Output |  |  |  |
|  | 20 |  | RESERVED |  |  |  |  |  | Reserved |  |  |  |
| ※The Host Serial Data I/O is nominally a CMOS logical high +5VDC. ※The Host Serial Data Input A (pin#12) suggest to an active high (ex.100Kohm serial to +Vcc) when not in use. |  |  |  |  |  |  |  |  |  |  |  |  |

USA, Canada and South America

ISHTRONICS, Inc.

3620 Kennesaw, N. Ind. Pkwy

Kennesaw, GA 30144

Tel: (770) 590-1822  Fax: (770) 422-8408 e-mail: info@ishtronics.com

| TITLE: CERTIFICATION | SPECIFICATION | PART NO: 1100-1000 |
| --- | --- | --- |
|  |  | DOCUMENT NO: TFA0001 |
|  |  | PAGE: 4 of 4 |
| 6.Environmental Characteristics 6.1Mechanical Test 6.1.1Vibration Test The samples are in operational mode and subjected to vibration of 5 mm constant amplitude in frequency range 5 ~ 12.2 Hz and 3G acceleration in frequency range 12.2 ~ 500 Hz(in SINUS wave). The test is in all three(X,Y,Z) axis, one axis at a time. The frequency change is 1 octave/min. and running for 46 cycle on each axis. After test, the samples are normally powered up and running without any error message(in Demo mode). The test is reference in DIN EN 60068-2-6. 6.1.2 Shock Test The samples are in operational mode and subjected to shock in 50G peck value acceleration (half SINUS wave in 11 ms duration). The test is in all three(X,Y,Z) axis with both direction for each axis and 3 shocks per direction. After test, the samples are normally powered up and running without any error message(in Demo mode) or external change. The test is reference in DIN EN 60068-2-27. 6.2 Environmental Test 6.2.1 High Temperature Test The samples are in operational mode and subjected to increase the temperature from 25C to 85C for two hours; then keep the samples in 85C  3C continuously for 16 hours, the temperature change will not exceed 1C/min.. Set relative humidity in 50%. The samples are normally powered up and running without any error message(in Demo mode). The test is reference in IEC 68-2-2 85C class. 6.2.2 Low Temperature Test The samples are in operational mode and subjected to decrease the temperature from 25C to -40C for two hours; then keep the samples in -40C  3C continuously for 16 hours, the temperature change will not exceed 1℃/min. .Set relative humidity in 50%. The samples are normally powered up and running without any error message(in Demo mode) The test is reference in IEC 68-2-1 3K7 class. |  |  |